- FU618F产品样本

- FU618是采用网状钍钨阴

- 极、焊接鼠笼形栅极、同轴型

- 电极引出结构的金属陶瓷四极

- 管,最大输出功率5.3kW，最

- 高工作频率150MHz。可与4CX3000A互换使用。

- 基本特性

- 1.1 阴极特性

- 加热方式                                        直热式

- 加热电压 （ Uf ，AC或DC）                      9V

- 加热电流 （If ）                                约41.5A

  - 静态特性

- 内放大系数                                       5.5

- 跨导                                           35mA/V

- 输入电容                                        130pF

- 输出电容                                        12.5pF

- 跨路电容                                        1.0pF

- 1.3 工作特性

- 工作位置                                      阳极朝上或朝下

- 质量                                           2.5kg

- 阳极冷却方式                                   强迫风冷

- 最小风量                                       18000L/min

- 出风口最高温度                                 100℃

- 封接处最高温度                                 250℃

- 引出端最高温度                                 150℃

- 管壳最高温度                                   250℃

2  额定最大值

频率                    f         150         MHz

阳极电压                Ua        7           kV

阳极电流                Ug1       2           A

阳极耗散功率            Pa        3.5          kW

一栅极耗散功率          Pg1        50          W

二栅极耗散功率          Pg2       175          W

3 典型应用

阳极电压                Ua      5.3         kV

二栅电压                Ug2     850         V

阳极电流                Ia       1.7         A

输出功率                Pout        5.3        kW

4 产品外形图

FU618F  TETRODE

- The Xuguang’s FU618F could be used instead of the 4CX3000A.

# 1  General Characteristics

- 1.1  Cathode Characteristics

- Heating 	Direct

- Heating voltage (Uf) 	9V

- Heating current (If)	Appr.41.5A

- 1.2  Feature Characteristics

- Internal Amplification Factor	5.5

- Transconductance	35mA/V

- Input 	130pF

- Output	12.5pF

- Transfer	1.0pF

- 1.3  Operating Characteristics

Operating position                        Vertical,anode up or down

Weight                                       2.5kg

Anode cooling                                 Forced air

Air flow,min                                  18000L/min

Temperature at the air outlet,max                   100℃

Temperature at the seal,max                       250℃

Temperature at the terminal,max                   150℃

Temperature on the surface of the tube,max           250℃

# 2  Maximum Ratings

- Frequency                    f         150         MHz

Anode DC Voltage             Ua         7          kV

Anode Dissipation              Pa         3.5         kW

Control-grid Dissipation         Pg1        50          W

Screen-grid Dissipation          Pg2       175          W

# 3 Typical Application

Anode DC Voltage             Ua        5        kV

Screen-grid Current            Ug2       850       V

Anode Current                Ia         1.7       A

Output Power                 Pout        5.3      kW

# 4 Product Outline Drawing